Common for all Instructions:

| Time | RTL | Bus | Active Signals |
| --- | --- | --- | --- |
| T0 | AR ← PC | PC (011) | LD_AR = 1 |
| T1 | M[AR], PC ← PC + 1 | Memory (001) | LD_IR=1, MEM_READ=1, INC_PC=1 |
| T2 | AR ← IR[11:4] | IR (110) → bits [11:4] | LD_AR=1 · decoder gets IR[14:12] |

LDA (D2) _ Load AC from Memory

| Time | RTL | Bus | Active Signals |
| --- | --- | --- | --- |
| T3 | DR ← M[AR] | Memory (001) | LD_DR=1, MEM_READ=1 |
| T4 | AC ← DR, SC ← 0 |  | LD_AC=1, ALU=001, CLR_SC=1 |

ADD (D1) _ Add Memory to AC

| Time | RTL | Bus | Active Signals |
| --- | --- | --- | --- |
| T3 | DR ← M[AR] | Memory (001) | LD_DR=1, MEM_READ=1 |
| T4 | AC ← AC + DR, SC ← 0 |  | LD_AC=1, ALU=010, CLR_SC=1 |

XOR (D0) _ XOR Memory with AC

| Time | RTL | Bus | Active Signals |
| --- | --- | --- | --- |
| T3 | DR ← M[AR] | Memory (001) | LD_DR=1, MEM_READ=1 |
| T4 | AC ← AC XOR DR, SC ← 0 |  | LD_AC=1, ALU=111, CLR_SC=1 |

DIV (D5) _ Divide AC by Memory

| Time | RTL | Bus | Active Signals |
| --- | --- | --- | --- |
| T3 | DR ← M[AR] | Memory (001) | LD_DR=1, MEM_READ=1 |
| T4 | AC ← AC / DR, SC ← 0 |  | LD_AC=1, ALU=101, CLR_SC=1, SWAP=1 |

ALU normally computes A/B = DR/AC. SWAP signal (=D5·T4) activates the MUX to swap: A=AC, B=DR

STA (D3) _ Store AC to Memory

| Time | RTL | Bus | Active Signals |
| --- | --- | --- | --- |
| T3 | M[AR] ← AC, SC ← 0 | AC (101) | MEM_WRITE=1, CLR_SC=1 |

BUN (D4) _ Branch unconditionally

| Time | RTL | Bus | Active Signals |
| --- | --- | --- | --- |
| T3 | PC ← AR, SC ← 0 | AR (010) | LD_PC=1, CLR_SC=1 |

ISZ (D6) _ Increment Memory and skip if zero

| Time | RTL | Bus | Active Signals |
| --- | --- | --- | --- |
| T3 | DR ← M[AR] | Memory (001) | LD_DR=1, MEM_READ=1 |
| T4 | DR ← DR + 1 |  | INC_DR=1 |
| T5 | M[AR] ← DR if (DR = 0): PC ← PC + 1 SC ← 0 | DR (100) | MEM_WRITE=1 INC_PC = Zero(DR) CLR_SC=1 |

Zero(DR) is the output of a 16-input NOR gate connected to DR. Zero(DR)=1 when DR = 0x0000.

Bus Select Lines

S2 = T2 + D3·T3 + D6·T5

S1 = T0 + T2 + D4·T3

S0 = T0 + T1 + (D0+D1+D2+D3+D5+D6)·T3

ALU

C2 = (D0 + D5) · T4

C1 = (D0 + D1) · T4

C0 = (D0 + D2 + D5) · T4

Loads and Incrementors

LD_AR = T0 + T2

LD_IR = T1

LD_DR = T3 · (D0 + D1 + D2 + D5 + D6)

LD_AC = T4 · (D0 + D1 + D2 + D5)

LD_PC = D4 · T3

INC_DR = D6 · T4

INC_PC = T1 + (D6 · T5 · Zero)

CLR_SC = T4·(D0+D1+D2+D5) + T3·(D3+D4) + D6·T5

NB. All undefined values of inputs are by default used as 0.

Memory read and write

MEM_READ = T1 + T3·(D0 + D1 + D2 + D5 + D6)

MEM_WRITE = D3·T3 + D6·T5